Mapping for a Cylindrical Micro-Resistive Well Detector

1. Group Members:

Alejandro Moreno Zuluaga, moreno2023@my.fit.edu

Summer Mueller, smueller2023@my.fit.edu

2. Faculty Advisors:

Pietro Iapozzuto, piapozzuto2015@my.fit.edu

Dr. Marcus Hohlmann, hohlmann@fit.edu

3. Clients:

Pietro Iapozzuto- Physics department

Dr. Marcus Hohlmann- Physics department

4. Milestone 3 Task Progress Matrix:

| Task | Completion | Alejandro | Summer | TO DO |
| --- | --- | --- | --- | --- |
| - Generate cylindrical mapping functions | 85% | 25% | 60% | We have to adjust the mapping for apv 9 and 13. |
| - Generate resolution plots for cylindrical detector | 70% | 45% | 25% | We have mathematical model of the grid of the detector. |
| - Validate cylindrical resolution plots against tracker resolution plots | 0% | 0% | 0% | We need resolution plots to properly validate the cylindrical detector. |

5. Discussion of task progress for M3

- Generate cylindrical mapping functions

The cylindrical mapping functions where found reading the documentation provided to use by the physics students, We were able to see that the (u, v) split of the strips per each apv was what we expected. Meaning that apv 8 would get more u’s than v’s apv 9 should get a similar amount of u’s and v’s then 10 should get more v’s than u’s and the inverse should happen in 12, 13

- Generate resolution plots for cylindrical detector

To generate the resolution plots for the cylindrical detector we needed to understand that the grid used in the detector is rotated. Complicating things a little bit. First we need to understand that the input we can get from the detector is a strip which can be understood as an axis-aligned point at x = 0 or x = 364, so a point in the detector is (0, strip) or (364, strip). Our origin point is the 0, 0 strip in apv 8. When we split between u and v strips we get a a point and some direction, and with that we can define a line that goes through that point and in the direction of the direction vector. We get a (u, v) pair and check where they intersect, if the intersection is within the area of the detector it is a valid hit, otherwise it is not valid.

- Validate cylindrical resolution plots against tracker resolution plots

The validation for the cylindrical resolution plots weren’t able to be done, because we’re still uncertain about the mapping.

6. Discussion of team member contribution

Alejandro- I contributed to the beam reconstruction in the cylindrical detector laying out the mathematical modeling of the detector.

Summer- In terms of physical deliverables, I was responsible for developing the mapping functions for the apv strips and creating programs to test the logic before applying to the reconstruction. Lastly, I contributed to the M3 document and presentation.

7. Milestone 4 Task Matrix

| Task | Alejandro | Summer |
| --- | --- | --- |
| - Finalize the cylindrical mapping, and beam reconstruction | 50% | 50% |
| - Start adapting the pipeline for multiple root files | 50% | 50% |

8. Discussion of tasks planned for M4

- Finalize the cylindrical mapping will allow us to start building efficiency plots and better resolution plots, and we can start focusing on building a solid pipeline to process many root files.

- At the same time we need to start making a pipeline that goes from root files to graphs.

9-10. See faculty meetings and feedback below

11. Dates and Meetings with Faculty:

- March 30th, 2026

- April 9th, 2026

- April 13th, 2026

- April 15th, 2026

12. Faculty feedback on tasks for M3

Faculty Advisor Signature: _______________________________ Date: ________

Evaluation by Faculty Advisor

Faculty Advisor: detach and return this page to Dr. Chan (HC 209) or email the scores to pkc@cs.fit.edu

Score (0-10) for each member: circle a score (or circle two adjacent scores for .25 or write down a real number between 0 and 10)

| Summer | 0 | 1 | 2 | 3 | 4 | 5 | 5.5 | 6 | 6.5 | 7 | 7.5 | 8 | 8.5 | 9 | 9.5 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alejandro | 0 | 1 | 2 | 3 | 4 | 5 | 5.5 | 6 | 6.5 | 7 | 7.5 | 8 | 8.5 | 9 | 9.5 | 10 |

Faculty Advisor Signature: _______________________________ Date: __________